Hypertech Scan Tool

User Manual

Version 2.4

Contents

1.	Introduction	3

1.1	Overview	3

2.	CAN Bus Tools (CANBUS) 	4

2.1	CAN0: LCD	4

2.2	CAN0: Serial	4

2.3	CAN1: LCD	4

2.4	CAN1: Serial	4

3.	Vehicle Tools (VEHTOOL)	5

3.1	PID Scan	5

3.2	PID Stream	5

3.3	PID Gauges	5

3.4	VIN	5

3.5	Scan DTC 	6

3.6	Clear DTC 	6

4.	Polaris RZR Tools (RZRTOOL)	6

4.1	Scan DTC	6

4.2	Clear DTC	6

5.	Extra Functions (EXTRAFN)	7

5.1	Disable AFM	7

5.2	Disable Auto Start / Stop	7

5.3	CAN Log	7

5.4	PCAN Log	7

6.	Tool Settings (SETTING)	8

6.1	CAN0 Filter / Mask	8

6.2	CAN1 Filter / Mask	8

6.3	CAN0 Set Baud Rate	8

6.3	CAN1 Set Baud Rate	8

7.	Creating New Functions	9

7.1	IDE	9

7.2	Adding Functions 	9

7.3	Hardware API	10

Introduction

The Hypertech scan tool is a multi-purpose device designed to assist engineers with data collection and other commonly used CAN Bus tools. All functions are organized into five pages. Each chapter will document the available page functions.

Figure 1 - Hypertech Scan Tool

2. CAN Bus Tools (CANBUS)

Read out CAN Bus traffic to the LCD or serial out

Figure 2 – CANBUS Page.

2.1 CAN0: LCD

Scrolls CAN Bus traffic across the screen from CAN0. Useful for detecting traffic and reading out data from non-vehicle traffic. The maximum write speed after the buffer is full is approximately 1 message every 30ms. For a vehicle CAN Bus dropped messages will occur.

2.2 CAN0: Serial

Read out CAN Bus traffic on serial out. Can be used with multiple terminal Apps. Arduino IDE is recommended for use. Fast enough to be used on a vehicle’s CAN Bus without dropping messages.

2.3 CAN1: LCD

Scrolls CAN Bus traffic across the screen from CAN0. Useful for detecting traffic and reading out data from non-vehicle traffic. The maximum write speed after the buffer is full is approximately 1 message every 30ms. For a vehicle CAN Bus dropped messages will occur.

2.4 CAN1: Serial

This function works differently from the rest. Traffic received from CAN1 is sent to serial out and forwarded while CAN0 traffic is only forwarded. This allows for the user to create a log of CAN Bus messages sent out from a device. Useful for reverse engineering.

3.	Vehicle Tools (VEHTOOL)

General vehicle diagnostic tools

Figure 3 - Vehicle Tool Page

3.1 PID Scan

Scan for all available SAE vehicle PIDs. Results are saved to the micro SD card in a folder with the last 8 characters of the vehicle’s VIN. In the folder, file LOG.txt will be created and contain the full vehicle VIN and a CAN Bus recording of received PID messages. A list of vehicle PIDs will be saved in a PID.txt file.

3.2 PID Stream

This function requires the user to first run PID Scan or an alert will popup informing the user it has not yet been run. PID stream provides the user with a scrollable list of available SAE vehicle PIDS. The user can select a PID and press the Stream PID button to obtain 5 samples of that PID saved to the LOG.txt file.

3.3 PID Gauges

This was an experimental function that contains 4 gauges updated in real time when connected to a vehicle or ECU emulator.

3.4 VIN

Displays the current connected vehicle’s VIN on the LCD screen.

3.5 Scan DTC

This function is currently in development and will be included in a future release.

3.6 Clear DTC

Clear any DTCs stored in the vehicle’s ECU. Universal support for all vehicles.

4.	Polaris RZR Tools (RZRTOOL)

Tools for the Polaris RZR line of sport utility vehicles

Figure 4- Polaris RZR Tools

4.1	Scan DTC

This function has not yet been ported over from the Polaris RZR Dongle project

4.2	Clear DTC

Clears any DTCs stored in the Polaris RZR ECU

5.	Extra Functions (EXTRAFN)

The extra functions page is the ideal spot for quick scripted test functions.

Figure 5- Extra Functions

5.1	Disable AFM

Testing code for disabling AFM in 2016-2021 GMC Trucks

5.2	Disable Auto Start / Stop

Testing code for disabling Auto Start / Stop in 2016-2021 GMC Trucks

5.3	CAN Log

Reads out data captures to the CAN Bus in the format generated from this device CAN Bus serial out. Save the file as “readcan.txt” on the Scan Tools SD card.

5.4	PCAN Log

Reads out data captures to the CAN Bus in the format generated from a PCAN CAN Bus capture. Edit out the heading from the .TLC data capture file and save as “readcan2.txt” on the Scan Tool’s SD card.

6.	Tool Settings (SETTING)

Change global settings for the scan tool

Figure 6- Tool Settings

6.1 CAN0 Filter / Mask

Allows the filter mask to be manually set for CAN0.

6.2 CAN1 Filter / Mask

Allows the filter mask to be manually set for CAN1.

6.3 CAN0 Set Baud Rate

Set CAN0 baud rate.

6.3 CAN1 Set Baud Rate

Set CAN1 baud rate.

7.	Creating New Functions

7.1 IDE

The program was developed using Visual Studios 2018 with vMicro extension to interface the Duo directly from Visual Studios. Other software that can be used: Visual Studio Code with Platform IO extension or the Arduino IDE (Not recommended).

7.2 Adding Functions

Each page is preprogrammed to allow 8 functions and has an associated draw page and page buttons. Activating requires uncommenting code. Adding function to EXTRAFN page example:

- Uncomment the button in the draw function:

- Next uncomment the button in extraFNButtons(). Buttons of the left side of the LCD have even page numbers and buttons on the ride odd. Page numbers are preassigned, do not change.

- In this case page 32 is used. Find 32 in pageControl().

- Add any content you want drawn for your function in the if statement, it will only run once. The menu sidebar is only drawn one time at startup. Avoid drawing over it unless you plan to call the side menu again. Any function buttons you want continuously called add below the if statement. All functions should be non-blocking to allow background functions to run.

- Background processes can be called outside the GUI in the main loop.

7.3	Hardware API (Partial list)

Send CAN Bus messages:

can1. sendFrame(uint32_t id, byte* frame)

Receive CAN Bus messages:

can1. getFrame(buff& msg, uint32_t& id, uint8_t channel)

Set filter and mask:

can1. setFilterMask0(uint32_t filter, uint32_t mask)

can1. setFilterMask1(uint32_t filter, uint32_t mask)

Set baud rate:

can1. setBaud0(uint32_t Baud)

can1. setBaud1(uint32_t Baud)

Read in file:

sdCard. readFile(char* filename, uint8_t* arrayIn)

Write to file:

sdCard. writeFile(char* filename, String incoming)

sdCard. writeFile(char* filename, int incoming, int base)

Delete file:

sdCard. deleteFile(char* filename)